運算放大器 (Operational Amplifiers)

低壓通用運算放大器 (Low Voltage General-Purpose Operational Amplifier)

| Part Number | Status | Channels | Supply Voltage(Min) | Supply Voltage(Max) | Iq per Channel | GBW(Typ) | Slew Rate (Typ)(V/µs) | Rail Rail In | Rail Rail OUT | VOS(Max) | Offset Drift (Typ)(uV/°C) | lb(typ) (pA) | Vn(Tpy) (nV/√Hz) | Peak Noise at 0.1 to 10Hz(μVpp) | AOL | CMRR | PSRR | Ishort(Typ) (mA) | ESD | Operating Temperature Range(°C) | Package | Competitve Product |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GTV321 | Production | 1 | 2.5 | 5.5 | 90µA | 1Mhz | 0.7V/µs | YES | YES | ±4mV | ±0.22µ V/°C | ±30pA | 35nV/√Hz | 14.52µVpp | 120dB | 130dB | 120dB | 70mA | 3.5KV | -40-125°C | SC70-5, SOT23-5 MSOP8,SOP8 | LMV321 |
| GTV358 | Production | 2 | 2.5 | 5.5 | 90µA | 1Mhz | 0.7V/µs | YES | YES | ±4mV | ±0.22µ V/°C | ±30pA | 35nV/√Hz | 14.52µVpp | 120dB | 130dB | 120dB | 70mA | 3.5KV | -40-125°C | SOP8,MSOP8,TTSOP8,DFN2x2-8L, DFN3x3-8L | LMV358 |
| GTV324 | Production | 4 | 2.5 | 5.5 | 90µA | 1Mhz | 0.7V/µs | YES | YES | ±4mV | ±0.22µ V/°C | ±30pA | 35nV/√Hz | 14.52µVpp | 120dB | 130dB | 120dB | 70mA | 3.5KV | -40-125°C | SOP14,TSSOP14 | LMV324 |
| GTV621 | Production | 1 | 2.5 | 5.5 | 700µA | 6.5Mhz | 2.9V/µs | YES | YES | ±4mV | ±2µ V/°C | ±10pA | 130nV/√Hz | / | 110dB | 92dB | 93dB | ±70mA | 7KV | -40-125°C | SC70-5,SOT23-5,MSOP8,SOP8 | / |
| GTV622 | Production | 2 | 2.5 | 5.5 | 700µA | 6.5Mhz | 2.9V/µs | YES | YES | ±4mV | ±2µ V/°C | ±10pA | 130nV/√Hz | / | 110dB | 92dB | 93dB | ±70mA | 7KV | -40-125°C | SOP8,MSOP8,TTSOP8,DFN2x2,DFN3x3-8L | / |
| GTV624 | Production | 4 | 2.5 | 5.5 | 700µA | 6.5Mhz | 2.9V/µs | YES | YES | ±4mV | ±2µ V/°C | ±10pA | 130nV/√Hz | / | 110dB | 92dB | 93dB | ±70mA | 7KV | -40-125°C | SSOP14,TSSOP14 | / |
| GTV721 | Production | 1 | 2.2 | 5.5 | 635µA | 13Mhz | 6.5V/µs | YES | YES | ±4mV | ±1µ V/°C | ±25pA | 88nV/√Hz | / | 95dB | 85dB | 85dB | ±80mA | 7.5KV | -40-125°C | SC70-5,SOT23-5,MSOP8,SOP8 | / |
| GTV722 | Production | 2 | 2.2 | 5.5 | 635µA | 13Mhz | 6.5V/µs | YES | YES | ±4mV | ±1µ V/°C | ±25pA | 88nV/√Hz | / | 95dB | 85dB | 85dB | ±80mA | 7.5KV | -40-125°C | SOP8,MSOP8,TSSOP8, ,DFN2x2-8L, DFN3x3-8L | / |
| GTV724 | Production | 4 | 2.2 | 5.5 | 635µA | 13Mhz | 6.5V/µs | YES | YES | ±4mV | ±1µ V/°C | ±25pA | 88nV/√Hz | / | 95dB | 85dB | 85dB | ±80mA | 7.5KV | -40-125°C | SSOP14,TSSOP14 | / |
| GT8391 | Preview | 1 | 2.2 | 5.5 | 2mA | 28Mhz | 9V/µs | YES | YES | ±2.5mV | ±1.3µ V/°C | ±25pA | 19.4nV/√Hz | / | 90dB | 77dB | 90dB | 100mA | 6.5K | -40-125°C | SC70-5,SOT23-5,MSOP8,SOP8 | / |
| GT8392 | Preview | 2 | 2.2 | 5.5 | 2mA | 28Mhz | 9V/µs | YES | YES | ±2.5mV | ±1.3µ V/°C | ±25pA | 19.4nV/√Hz | / | 90dB | 77dB | 90dB | 100mA | 6.5K | -40-125°C | SOP8,MSOP8,TSSOP8, DFN2x2-8L, DFN3x3-8L | / |
| GT8394 | Preview | 4 | 2.2 | 5.5 | 2mA | 28Mhz | 9V/µs | YES | YES | ±2.5mV | ±1.3µ V/°C | ±25pA | 19.4nV/√Hz | / | 90dB | 77dB | 90dB | 100mA | 6.5K | -40-125°C | SOP14,TSSOP14 | / |