運算放大器 (Operational Amplifiers)

高壓通用運算放大器 (High Voltage General-Purpose Operational Amplifier)

| Part Number | Status | Channels | Supply Voltage(Min) | Supply Voltage(Max) | Iq per Channel | GBW(Typ) | Slew Rate (Typ)(V/µs) | Rail Rail In | Rail Rail OUT | VOS(Max) | Offset Drift (Typ)(uV/°C) | lb(typ) (pA) | Vn(Tpy) (nV/√Hz) | Peak Noise at 0.1 to 10Hz(μVpp) | AOL | CMRR | PSRR | Ishort(Typ) (mA) | ESD | Operating Temperature Range(°C) | Package | Competitve Product |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GT321 | Production | 1 | 3 | 36 | 117uA | 1Mhz | 1V/μs | NO | YES | ±3mA | ±3µ V/°C | ±30pA | 35nV/√Hz @1MHz | / | 100dB | 95dB | 100dB | 45mA | 3KV | -40-125°C | SOT23-5,SOP-8,MSOP-8 | LM321 |
| GT358 | Production | 2 | 3 | 36 | 117uA | 1Mhz | 1V/μs | NO | YES | ±3mA | ±3µ V/°C | ±30pA | 35nV/√Hz @1MHz | / | 100dB | 95dB | 100dB | 45mA | 3KV | -40-125°C | SOP-8,MSOP-8,DFN2x2-8L | LM358 |
| GT324 | Production | 4 | 3 | 36 | 117uA | 1Mhz | 1V/μs | NO | YES | ±3mA | ±3µ V/°C | ±30pA | 35nV/√Hz @1MHz | / | 100dB | 95dB | 100dB | 45mA | 3KV | -40-125°C | SOIC-14,TSSOP-14 | LM324 |